"Microcontrollers Explained: Build a Legal WiFi Detector with ESP32" What is a Microcontroller?

1. Definition

A microcontroller is a small computer built into a single integrated circuit (chip). It contains:

- A processor (CPU)

- Memory (RAM and Flash)

- Input/Output (I/O) pins

- Communication modules (WiFi, Bluetooth, UART, etc.)

Unlike a desktop computer, a microcontroller is designed to control specific tasks inside electronic systems.

Examples used in this module:

- ESP32 (by Espressif Systems)

- M5Stack Core (ESP32-based development kit by M5Stack)

2. What Does a Microcontroller Do?

A microcontroller:

- Reads inputs (buttons, sensors, WiFi signals)

- Processes data using programmed instructions

- Controls outputs (LEDs, screens, motors) In this project:

- The ESP32 scans WiFi networks

- It counts detected networks  It turns on:

o  Green LED (Normal) o  Yellow LED (Caution) o  Red LED (Alert)

It is acting as a decision-making controller

- Microcontroller vs Microprocessor

| Feature Microcontroller | Microprocessor |
| --- | --- |
| Purpose Embedded control | General computing |
| Memory Built-in | External required |
| Example ESP32 | Laptop CPU |

Used in	IoT devices, robotics PCs, servers Microcontrollers are used in:

  - Washing machines

  - Cars

  - Smart TVs

  - Medical devices

  - IoT systems

- Internal Components of ESP32 The ESP32 contains:

  - Dual-core processor

  - WiFi module

  - Bluetooth module

  - GPIO pins (General Purpose Input Output)

  - ADC (Analog to Digital Converter)

  - Flash memory

This makes it ideal for:

  - Wireless detection

  - Sensor monitoring

  - Smart automation

- Why ESP32 is Used in This Project We use the ESP32 because:

  - It has built-in WiFi

  - It supports scanning nearby networks

  - It can control LEDs

  - It is affordable and educational

  - It supports programming in Arduino IDE Most importantly:

It performs PASSIVE detection only. It does NOT transmit interference.

- Step-by-Step: How the Microcontroller Works in This Project

  - Power is supplied through USB.

  - The program starts running.

  - WiFi scan begins.

  - Number of networks is counted.

  - Conditional logic checks thresholds.

  - LED color output is activated.

  - Process repeats every 5 seconds.

This is called a control loop.

- Teaching Explanation (Instructor Notes) When explaining to students:

  - Compare a microcontroller to a “tiny brain” of a device.

  - Explain that it follows instructions written in code.

  - Emphasize embedded systems concept.

  - Reinforce that detection is legal, jamming is illegal.

# Student Module

## Wireless Interference Detection Project

- LEGAL NOTICE – DETECTION ONLY: This module describes a wireless interference

DETECTION

- system. It does NOT transmit signals and MUST NOT be modified into a jammer. Jamming  communications is illegal in most countries and may result in severe penalties.

## What You Will Learn

- How WiFi scanning detects nearby networks.

- Difference between detection and illegal jamming.

- How LEDs/LCD provide status feedback.

## Color Meaning

- GREEN Normal wireless environment

- YELLOW Caution – increased signal activity

- RED Alert – strong interference detected

Lab Activity

- Measure signal levels in 3 different locations.

- Record number of networks detected.  Explain why results differ.

## Worksheet Questions

- Why is wireless jamming illegal?

- What does GREEN indicate?

- What environment produced RED? Why?

# ExperimentActivity

🔧 Step-By-Step Build Procedure (Student Instructions)

Before You Start

Make sure you have the following materials:

• ESP32 development board
• Breadboard
• 3 LEDs (Green, Yellow, Red) 
• 3 × 220Ω resistors
• Jumper wires
• USB cable
• Computer with Arduino IDE installed

Step 1 – Insert the ESP32 into the Breadboard

- Place the ESP32 across the center gap of the breadboard.

- Make sure both rows of pins are accessible.

- Do NOT connect USB power yet.

Step 2 – Connect the GREEN LED (GPIO 25)

- Insert the long leg (anode) of the green LED into the breadboard.

- Insert the short leg (cathode) into a different row.

- Connect:
• GPIO 25 → one end of a 220Ω resistor
• Other end of resistor → long leg of green LED
• Short leg of green LED → GND pin on ESP32

The green LED represents NORMAL condition.

Step 3 – Connect the YELLOW LED (GPIO 26)

- Insert the yellow LED into the breadboard.

- Connect:
• GPIO 26 → 220Ω resistor → long leg of yellow LED
• Short leg → GND

The yellow LED represents CAUTION condition.

Step 4 – Connect the RED LED (GPIO 27)

Insert the red LED into the breadboard.

Connect:

• GPIO 27 → 220Ω resistor → long leg of red LED

• Short leg → GND

The red LED represents ALERT condition. Step 5 – Check All Connections

Before powering the board:

• Each LED must have its own 220Ω resistor
• All short LED legs must go to GND
• No wires should touch incorrectly
• Verify GPIO numbers are correct

This prevents short circuits and damage.

Step 6 – Connect and Power the ESP32

- Connect the ESP32 to your computer using USB.

- Wait for the computer to detect the device.

Step 7 – Configure Arduino IDE

- Open Arduino IDE.

- Go to Tools → Board → Select “ESP32 Dev Module”.

- Go to Tools → Port → Select the correct COM port.

- Paste the provided code into the editor.

Step 8 – Upload the Program

- Click Upload.

- If upload fails:
• Hold the BOOT button while clicking Upload.
• Release when uploading begins.

Wait until “Done Uploading” appears.

Step 9 – Test the System

- Open Serial Monitor.

- Set baud rate to 115200.

- Observe the number of WiFi networks detected.

- Observe which LED turns on.

Move to different locations to compare results.

code

#include <WiFi.h>

int G = 25;

int Y = 26;

int R = 27;

void setup() {

Serial.begin(115200);

pinMode(G, OUTPUT);

pinMode(Y, OUTPUT);

pinMode(R, OUTPUT);

WiFi.mode(WIFI_STA);

WiFi.disconnect();

delay(100);

}

void loop() {

Serial.println("Scanning...");

int n = WiFi.scanNetworks();

Serial.print("Networks detected: ");

Serial.println(n);

if (n > 5) {

digitalWrite(R, HIGH);

digitalWrite(Y, LOW);

digitalWrite(G, LOW);

}

else if (n > 2) {

digitalWrite(Y, HIGH);

digitalWrite(R, LOW);

digitalWrite(G, LOW);

}

else {

digitalWrite(G, HIGH);

digitalWrite(R, LOW);

digitalWrite(Y, LOW);

}

delay(5000);

}